# SOP: 1Meter MAK Field Visit Update

## Purpose

This update replaces the earlier full-fleet cleanup sequence. The team is already en route, and this version is based on live AWS IoT, DynamoDB, and 1PDB checks from the morning of 2026-03-16.

Today’s goal is to restore or confirm live telemetry and lock down the true device identities. Do not treat this visit as an OTA campaign or a blanket reflash of every board.

## Live Summary

- 23022696 at 0025MAK should now be running as OneMeter16.

- AWS already has a valid OneMeter16 thing, cert, and group membership, but there has been zero DynamoDB telemetry under OneMeter16.

- 0026MAK is still mapped in CC to 23022684, but the live cloud telemetry is coming from 23021847 using OneMeter14.

- 23022646 is publishing as OneMeter5 and last reported at about 2026-03-15 22:45 SAST.

- 23021847 is publishing as OneMeter14 and last reported at about 2026-03-15 22:57 SAST.

- 23022628 (OneMeter18), 23022673 (OneMeter13), and 23022667 (OneMeter44) are alive today.

- AWS OTA status right now:

  - OneMeter44: SUCCEEDED

  - OneMeter16: QUEUED

  - OneMeter5, OneMeter13, OneMeter14, OneMeter18: IN_PROGRESS

- Weak connection at MAK can cause OTA downloads to restart from zero. Do not use OTA as the primary repair path during this visit.

## Immediate Priorities

- Get the first successful cloud publish from 23022696 under OneMeter16.

- Confirm whether the board now serving 0026MAK is really 23021847.

- Check the neighboring boards 23022646 and 23021847, since both stopped around 22:xx last night.

- Leave currently healthy boards alone unless the local console or cloud identity contradicts what is expected.

## What Not To Assume

- Do not assume s3://1pwr-device-certs/ has the cert folders. It currently contains only a stale device-map.json.

- Do not flash 23022696 back to OneMeter15. The target identity now is OneMeter16.

- Do not start another OTA just because one is queued or in progress. Stable telemetry comes first.

- Do not reuse one cert folder on multiple physical boards.

## Laptop And Spares Check

- Use the local cert folders already on the field laptop.

- Make sure these bundles are locally available before flashing anything:

  - onemeter13

  - onemeter14

  - onemeter18

  - onemeter44

  - onemeter16 if 23022696 is staying on OneMeter16

- If the onemeter16 bundle is not available locally, do not improvise with onemeter15. Stop and escalate before reflashing 23022696.

- Bring:

  - spare PCB for 23022696

  - spare DDS8888

  - labels or marker for enclosure labeling

  - known-good USB cable and laptop power

## Field Execution Order

### 1. 0025MAK / 23022696 / target identity OneMeter16

Goal:

- Get the first successful cloud publish under OneMeter16.

Steps:

- Record the physical serial printed on the installed board.

- Open the serial monitor and confirm the device boots as OneMeter16.

- If a reflash is needed, use:

- python auto_flash.py --cert-folder onemeter16 --port COMXX

- Confirm the cert bundle in use is onemeter16, not the old onemeter15 bundle.

- Watch for:

  - Wi-Fi association

  - MQTT connect success

  - first publish

- Reboot once after flash or power-cycle and wait for a fresh publish before leaving.

- Ignore the queued OTA until the board is stably publishing.

- If it does not publish, capture the exact serial-console failure and send it back before moving on.

### 2. 0026MAK / target identity OneMeter14

Goal:

- Confirm whether the live board is 23021847 or 23022684.

Steps:

- Record the physical serial on the board currently installed at 0026MAK.

- Confirm the serial monitor shows OneMeter14.

- If a reflash is needed, use:

- python auto_flash.py --cert-folder onemeter14 --port COMXX

- Compare the physical serial with the cloud-visible serial:

  - cloud currently shows 23021847

- If the installed board is 23021847, keep that identity, label it clearly, and report that back so CC can be remapped remotely.

- If the installed board is 23022684 but cloud still only shows 23021847, stop and escalate. That is a serial or identity mismatch, not a normal CC ingestion problem.

- Confirm a fresh publish after reboot.

### 3. Neighbor check: 23022646 and the 0026MAK board

Goal:

- Confirm whether the near-simultaneous cutoff last night was a local comms or power issue.

Steps:

- At 23022646, confirm the console identity is OneMeter5.

- Confirm both boards have power and stable local connectivity.

- Check signal quality and any obvious local network weakness before changing more firmware.

- Reboot and confirm fresh cloud publish from both boards.

- If both fail together again, treat it as a local comms or environment issue first, not a backend issue.

### 4. Leave the healthy boards mostly untouched

- 23022628 / OneMeter18: live today. Do not reflash unless the local console or cloud identity says it is wrong.

- 23022673 / OneMeter13: live today. Do not replace hardware just because low load makes the percent deviation noisy.

- 23022667 / OneMeter44: gateway is live. Touch it only last and only if needed.

- 23022613: non-customer node. Cloud is also seeing OneMeter44 from this serial, so if you encounter it, record the physical serial and leave it for later cleanup instead of creating more identity churn today.

## Verification Checklist

After each device you touch, verify all of the following:

- Physical serial is recorded.

- Serial monitor shows the intended Thing Name.

- Device reports fresh telemetry after reboot.

- The enclosure is labeled with:

  - serial

  - Thing Name

  - account or role

- If the device does not publish, capture the exact console output before leaving that location.

## Remote Follow-Up After Visit

- Report whether 0026MAK should be remapped from 23022684 to 23021847.

- Send the exact cert folders used on site for secure backup. The S3 bucket is not currently a reliable cert source of truth.

- If S3 is used for backup after the visit, populate the actual cert bundles:

- cd onepwr-aws-mesh/main/certs
aws s3 sync . s3://1pwr-device-certs/certs/ --region us-east-1

- Only after stable telemetry and confirmed identities should the next OTA cleanup be attempted.

## Success Criteria For Today

- 23022696 publishes under OneMeter16 at least once.

- The physical serial at 0026MAK is confirmed and matches or explains the cloud identity.

- 23022646 and the 0026MAK board are confirmed alive, or a concrete local failure is captured.

- Healthy boards are not churned unnecessarily.

- No new OTA is started on unstable links.

## Operational Notes

- The best comparison meters to rely on after cleanup should still be:

  - 0005MAK

  - 0119MAK

  - 0045MAK with low-load caution

- 0025MAK and 0026MAK should rejoin the clean validation set only after stable telemetry is confirmed.

- OTA is technically proven, but MAK connectivity is still weak enough that it should not drive today’s field decisions.