# SOP: 1Meter MAK Next Steps

## Purpose

This SOP gives the field team a clear next-action plan for the deployed MAK 1Meter fleet after the latest review of:

- OTA readiness and cleanup

- device health

- 1Meter vs SparkMeter comparison quality

This document assumes no newer field intervention has happened since the latest verified notes used to prepare it.

## Current Summary

- OTA is proven end-to-end:

  - bench OTA succeeded

  - first field OTA succeeded on one device

- The 1Meter vs SparkMeter comparison pipeline is broadly validated when the hardware is healthy.

- The immediate blockers are now:

  - wrong device identity / cert provisioning on part of the fleet

  - one likely PCB failure

  - one confirmed DDS8888 measurement failure

## Comparison Baseline

These were the last clean comparison totals before the later device failures:

| Account | 1M Serial | Total Deviation | Interpretation |
| --- | --- | --- | --- |
| 0025MAK | 23022696 | +2.3% | Good agreement |
| 0026MAK | 23022684 | +1.2% | Good agreement before later DDS8888 fault |
| 0119MAK | 23022646 | +4.8% | Good agreement |
| 0005MAK | 23022628 | -1.8% | Good agreement |
| 0045MAK | 23022673 | -6.6% | Acceptable, but low-load quantization makes percent error look worse |

Interpretation:

- Healthy 1Meter check meters are tracking SparkMeter closely enough to support continued validation work.

- 0045MAK should be treated as a low-load edge case, not as evidence that the comparison pipeline is broken.

- 0025MAK and 0026MAK are currently repair cases and should not be treated as clean validation meters until fixed.

## Current Device Status

| Serial | Target Thing Name | Account / Role | Current Known State | Required Action |
| --- | --- | --- | --- | --- |
| 23022696 | onemeter15 | 0025MAK | Repeat dropout, likely PCB / ESP32 issue | Replace PCB, then flash correct identity |
| 23022684 | onemeter14 | 0026MAK | DDS8888 current-sensing failure; also part of identity cleanup | Replace DDS8888, then flash correct identity |
| 23022628 | onemeter18 | 0005MAK | Metering looks healthy; identity needs correction | Flash correct identity |
| 23022646 | TBD final name; currently ExampleThing | 0119MAK | Metering looks healthy; naming still inconsistent | Decide permanent Thing name and align certs |
| 23022673 | onemeter13 | 0045MAK | Metering looks healthy; low-load customer | Confirm cert and group membership; no hardware swap planned |
| 23022667 | onemeter44 | Gateway | Gateway identity needs correction | Flash correct identity last |
| 23022613 | onemeter55 | Repeater / powerhouse | Infrastructure node, not a customer comparison meter | No SM comparison action needed |

## Pre-Visit Checklist

- Pull the latest cert folders from the centralized S3 bucket before leaving:

- aws s3 sync s3://1pwr-device-certs/certs/ ./certs/ --region us-east-1

- Confirm these cert folders are present before the visit:

  - onemeter13

  - onemeter14

  - onemeter15

  - onemeter18

  - onemeter44

- If onemeter13 or onemeter14 is missing locally, stop and escalate before the visit. Remote IoT cleanup depends on those folders being available.

- Bring the required spare parts:

  - spare PCB for 23022696

  - spare DDS8888 for 23022684

  - labels or marker for enclosure labeling

  - known-good USB cable and laptop power

- Bring the device map and do not reuse the same cert folder on multiple devices.

## Field Execution Order

### 1. Repair 23022696 at 0025MAK

Reason:

- This device is currently offline / dropping out.

- It is one of the most valuable validation meters when healthy.

Steps:

- Replace the PCB.

- Flash with the correct identity:

- python auto_flash.py --cert-folder onemeter15 --port COMXX

- Confirm the serial monitor shows the correct Thing Name.

- Confirm fresh telemetry returns after reboot.

### 2. Repair 23022684 at 0026MAK

Reason:

- DDS8888 current sensing is faulty.

- This device also needs identity correction.

Steps:

- Replace the DDS8888.

- Flash with the correct identity:

- python auto_flash.py --cert-folder onemeter14 --port COMXX

- Confirm current readings are no longer stuck near 0-6 mA during load.

- Confirm fresh telemetry returns after reboot.

### 3. Correct 23022628 at 0005MAK

Reason:

- This device appears healthy and is one of the best meters for ongoing comparison.

- Identity must be cleaned up before the next OTA cycle.

Steps:

- Flash with the correct identity:

- python auto_flash.py --cert-folder onemeter18 --port COMXX

- Confirm the serial monitor shows OneMeter18.

- Confirm telemetry resumes normally.

### 4. Confirm 23022673 at 0045MAK

Reason:

- This meter looks healthy, but it operates at very low load.

- It should remain in the comparison fleet, but low-load deviations should not trigger overreaction.

Steps:

- Confirm its cert / Thing registration is correct.

- Do not replace hardware unless a real comms or measurement issue is seen.

### 5. Decide permanent identity for 23022646

Reason:

- The meter at 0119MAK is useful for comparison, but the naming is still inconsistent.

Steps:

- Decide the permanent Thing Name before the next group OTA.

- Align cert folder, Thing registration, and enclosure label to that decision.

### 6. Correct gateway 23022667 last

Reason:

- This device is infrastructure, not a customer comparison meter.

- It should be touched last so mesh / gateway disruption happens only after the customer devices are done.

Steps:

- Flash with the correct identity:

- python auto_flash.py --cert-folder onemeter44 --port COMXX

- Confirm telemetry / gateway behavior returns before leaving site.

## Verification Checklist

After each flash or hardware swap, verify all of the following:

- Serial monitor shows the correct Thing Name.

- Device reports fresh telemetry after reboot.

- The enclosure is labeled with:

  - serial

  - Thing Name

  - account or role

- If the device is a customer check meter, note whether it should remain in the active SM-vs-1M comparison set.

## Remote Follow-Up After Visit

After the team uploads any missing cert folders to S3:

cd onepwr-aws-mesh/main/certs
aws s3 sync . s3://1pwr-device-certs/certs/ --region us-east-1

Remote follow-up should then be:

- Attach any missing certs for onemeter13 / onemeter14 in AWS IoT.

- Confirm the thing group contains the intended production devices.

- Only then prepare the next OTA campaign.

- Use a firmware version higher than the currently deployed field image for the next OTA push.

## Acceptance Criteria

The fleet is ready for the next structured OTA cycle when:

- No two physical devices share the same Thing identity.

- 0025MAK is back online and stable.

- 0026MAK reports realistic current again.

- 0005MAK, 0119MAK, and 0045MAK have confirmed identities and fresh telemetry.

- The gateway is corrected and still operational.

- The cert folders needed for future field work are centralized in S3.

## Operational Notes

- The best current comparison meters to rely on after cleanup should be:

  - 0005MAK

  - 0119MAK

  - 0045MAK with low-load caution

- 0025MAK and 0026MAK should only be reintroduced into the “clean validation” set after repair and post-repair verification.

- OTA should now be treated as technically proven, but not yet fleet-safe until identity and failing hardware are cleaned up.