Team, here is the current MAK action plan based on Motlatsi’s field report plus the live cloud check.

Current position: - work at MAK is not yet complete - 23022696 PCB was already replaced, and the target identity is now OneMeter16 - 0026MAK cloud data is currently coming from serial 23021847 as OneMeter14 - 23022684, 23022628, and 23022667 were already confirmed on console as OneMeter14, OneMeter18, and OneMeter44 - 23022646 Thing Name is now OneMeter5 - OneMeter13 and OneMeter14 cert folders were recreated locally - cert folders have NOT yet been backed up to S3 - gateway OTA succeeded, 23022696 is queued, and the rest are in progress - OTA needs stable connection, and MAK signal was weak, so OTA is not today’s priority

Main action order:

- 0025MAK / 23022696:

- do NOT flash back to OneMeter15

- confirm it boots as OneMeter16

- if reflash is needed, use onemeter16

- confirm Wi-Fi, MQTT connect, and first cloud publish before leaving

- 0026MAK:

- record the physical serial on the installed board

- confirm the console shows OneMeter14

- if the installed board is 23021847, keep it labeled that way and tell me so I can update CC mapping remotely

- if the installed board is 23022684 but cloud still shows 23021847, stop and send the exact situation

- 0119MAK / 23022646:

- confirm the console shows OneMeter5

- because 23022646 and the 0026MAK board stopped almost together last night, please treat this as a local comms / power check too

- Leave the currently healthy boards mostly alone unless local console or cloud identity says otherwise:

- 23022628 / OneMeter18 is live

- 23022673 / OneMeter13 is live

- 23022667 / OneMeter44 is live

Important: - use the local cert folders already on the laptop - do NOT rely on S3 for certs right now; the bucket currently only has device-map.json - do NOT reuse cert folders across devices - if 23022696 needs reflash, use onemeter16, not onemeter15 - get stable telemetry first, then OTA later

After each touched device, send back: - physical serial - Thing Name shown on console - whether telemetry returned - photo / screenshot of console if it did not publish

Full SOP is in: docs/SOP-1Meter-MAK-Next-Steps.md